|  | CFAS Enterprises Inc. After Market Utility Power Equipment Brokerage Mailto:Staff@CFASPower.com Turbine URL: http://CFASPower.com/Index.html Reciprocating URL: http://CFASPower.com/Recips.html |
| --- | --- |
| GTG_1320GR_FM6.doc |  |

2 x 1997 GEC Alsthom PG6541B 50Hz 38Mw Diesel Gas Turbine Generators

Asking Price:						USD 5.75MM Each Negotiable

Complete BOP Included

YOM:							 	1997

Operation:							Peakers

Starts:								NO.1 895 /  NO.2 869,

Total operating Times:				NO.1 5388 hours / NO.2 4939 hours.

Interim overhaul:						April and October of 2004

Condition:							Very Good

  - Gas Turbine Design Data

Manufacturer：						GEC Alsthom

Serial Number：						T766，T768

Type：								PG6541B

Gas Turbine Application：				Generator Drive

Cycle：								Simple

Compressor：Stages：				17

Speed：								5100 R.P.M.

Stages:								3

Speed：								5100 R.P.M.

Inlet Air Flow：						401400m3/H

Inlet Air Pressure Drop：				100mmh2o

Turbine Inlet Temperature： 			1104℃

Exhaust Gas Temperature：			533℃

Exhaust Pressure Drop：				30 Mmh2o

  - Gas Turbine Equipment Data Summary

Compressor Section

Number Of Compressor Stages：		17

Compression Ratio：					11.8

Compressor Type：					Axial Flow，Heavy Duty

Casing Split：						Horizontal Flange

Inlet Guide Vanes Type：				Variable

Turbine Section

Number Of Turbine Stages：			3 Single Shaft

Casing Split：						Horizontal Flange

Nozzles：							Fixed Area

Combustion Section

Type：								10 Multiple Combustors，Reverse Flow Design

Fuel Nozzles：						1 Per Combustion Chamber

Spark Plugs：						2 Electrode Type, Spring-Injected 												Self-Retracting（#1，#10）

Flame Detectors：					4，Ultra-Violet Type（#2，#3，#7，#8)

Bearing Assemblies

Quantity：							2

Lubrication：							Pressure Lubricated

No.1 Bearing Assembly

(Located In Inlet Casing Assembly)		Active And Inactive 	Thrust Journal，All 								Contained In One Assembly

Journal：							Elliptical

Active Thrust：						Tilting Pad，Self-Equalizing

Inactive Thrust：						Tapered Land No.2 Bearing Assembly (Located 									Exhaust Frame Assembly) ：Elliptical Journal

Starting System

Starting Device：						Diesel Starting Motor

Torque Converter：					Hydraulic With Integral Reversing Gear 									And Ratchet Device

Fuel System

Operating Type：						Light Distillate

Fuel Control Signal：					Speedtronic Control System

Fuel Pump：							Accessory Gear Driven，Continuous Output Screw 								Type Pump

Flow Divider （Starting Motor）：		In-Line Free Wheeling，10 Elements

Fuel Oil Stop Valve：					Electro-Hydraulic Servo-Control

Fuel Oil Filter

Lubrication System

Lubricant：							Petroleum Base

Total Capacity：						6500gallons（Approx.）

Main Lube Pump：					Shaft-Driven，Integral With Accessory Gear

Auxiliary Lube Pump：				AC Motor-Driven，Vertical，Submerged，								Centrifugal Type

Emergency Lube Pump：				DC Motor-Driven，Vertical，Submerged，								Centrifugal Type

Heat Exchange						Type，Oil Heat To Fresh Water；Quantity：1

Filter（S）：							Type，Full Flow With Transfer Valve；									Quantity，2（Duplex）；Cartridge Type：Five 								Micron Filtration Pleated Paper

Hydraulic Supply System

Main Hydraulic Supply Pump： 		Accessory Gear-Driven，Positive 									Displacement ，Axial Piston ）

Cooling Water System（In Closed Loop）

Pump：								Shaft-Driven

Atomizing Air System

Main Compressor：					Accessory Gear Driven

Centrifugal starting Compressor ：		Bolt Driven By The Starting Engine

Air Preschooler ：						Air-To-Water Heat Exchanger

Air Cleaner：							Separator

Cool down Mechanism：				Ratchet Device Integral With Torque Converter

Reduction Gear:						Speed Reduction Gear In Generator Package

Shaft Speed Ratio:					5100/3000

Control:								Mark V Speedtronic Solid State Electronic 								Control System

Protection：							Overspend，Over temperature，Vibration，								Flame Detection

Generator

Manufacturer ：						GEC Alsthom

Type：								T190-240

Phases:								3

Year Of Manufacture:					1996

Cooling By ：						Air

Rated Output：						44.882mva

Rated Voltage：						11000v

Rated Current：						2356a

Power Factor：						0.85

Ambient Temperature：				15℃

Speed：								3000rpm

Frequency：							50hz

Class Of Insulation：					F

Excitation：							249v 515a

Altitude：							1000m

Main Transformer：

Manufacturer：						Changzhou Transformer Company

Type：								SFZ8-50000/110

Phase：								3

Rated Frequency：					50hz

Rated Capacity：						50000kva

Rated Voltage：						115kv×1.25%/11

Connection：						YND11

Cooling：							Air Cooling

Lv Winding：

Voltage：							11000

Current：							2642A

No Load Current：					#1 0.17% #2 0.21%\

No Load Consume：					#1 32.40kw #2 30.8kw

Load Consume：						#1 204.73kw #2 209.02kw

110kv SF6 Breaker

Manufacturer：						GEC Alsthom

Type：								FXt11

Quantity：							5

U：

F：									50hz

In：									2000a

Isc ：									25ka

Tth：									3s