|  | CFAS Enterprises Inc. After Market Utility Power Equipment Brokerage Mailto:Staff@CFASPower.com Turbine URL: http://CFASPower.com/Index.html Reciprocating URL: http://CFASPower.com/Recips.html |
| --- | --- |
| GTG_1697MS_KongKG5_Mob.doc |  |

3 Unused Kongsberg/Viking KG5 3MW 50Hz or 60Hz 2740kW DF Mobil Power Plants

The Turbines were made in 1980 and have never been used requirement change. They are warehouse stored and in brand new condition. The Turbines are configured on trailers with complete set of control panel and alternator by Kobelco.

PRICED TO SELL: 		US $800,000 ALL 3 - FOB Kuching, Malaysia

Download KG5 Data Sheet

Download KG5 Specifications

Download KG5 Brochure

DESCRIPTION

The KG5 generating set is an industrial type electric power unit comprising an open-cycle, two-shaft gas turbine prime mover and a brushless open-ventilated generator rated to suit the gas turbine characteristics at various ambient conditions. Standard output voltages range from 380 V to 13.8 kV at 50 or 60 hertz.

SPECIFICATION

Output shaft speed:				1500 or 1800rev. min 150.60 Hz) as standard

Gearbox:					Epicyclical

Compressor pressure ratio:			6.3:1

Speed control characteristics:

Steady state speed control:			Max | 0.25%

TYPICAL ON-SITE PERFORMANCE DATA

(Sea level, 100 mm wg intake loss, 100 mm wg exhaust loss and 96% generator efficiency:

Ambient temp.                          		Max                                       Base Load

15UC (59°F)                            		3385 kVA                               3425 kVA

35°C I95°F)                              		3340 kVA                               2905 kVA

Max      Continuous power rating for operation up to 1500 hours per year

Output stated based on power factor:	0.8

Fuel consumption: 				1150 kg h (2535 Ib/h) (at 3000 kW. 16°C wi/ diesel fuel)

Heat Rate: 					11,747 KJ/KW-HR

** A regenerator can be installed when improved fuel economy is required *or continuous duty applications. Full load thermal efficiency can thus be increased to over 30% (corr to approx. 850 kg h diesel fuel at 3000 kW output).

Lube oil consumption: 			Approx. 0.07 kg/h (0.15 Ib/h)

STARTING

Starting is achieved by a starter motor connected to the gas generator rotor via a sprag-clutch in the auxiliary gearbox. Starting time to nominal speed with full load applied is 45 to 60 seconds depending on systems selected.

Materials

Component					Material

Turbine Wheel:				Waspaloy

Compressor wheel:				Titanium alloy.

Combustor:					Hastalloy X

Turbine Housing:				PE 10

Reduction Gear Casing:			Silumin

Lube oil Tank Foundation:			Mild Steel

Pressure Casing:				Coated Mild Steel

Power Turbine:				Waspaloy

CORROSION RESISTANCE

The complete generating set is designed for operation in all types of industrial environment and is also approved by the major marine classification societies for use on board ships. Special attention has been paid to corrosion resistance and sleeve bearings are employed to support ail rotating parts in the turbine engine.

DIMENSIONS

6820					2500		3230

Weight:

Complete generating set with accessories necessary to run (approx.:

21 003 kg	46 000 lb.

Combustor Head Complete:		123 kg		265 lb.

Auxiliary Gear complete:		243 kg		530 lb.

Gas Generator Rotor:			235 kg		5181b

Power Turbine Rotor:			73 kg		167 1b

Main Reduction Gear:			805 kg		1775 1b

Complete GT with Reduction Gear:	376D kg	8290 It)